Driver Notes

Wave/4 driver for NT 4.0

18 November 1998,    version 1.0

Please report problems with the driver via e-mail to support@gadgetlabs.com.

Start Up:

Please read these notes completely before installing the Wave/4 and the NT driver!

Windows NT is an advanced operating system that requires more technical expertise to administer and configure that Windows 95 or 98.    As such, the steps to installing and configuring an NT driver are considerably more involved than the steps for Win 95/98.    We recommend using the Wave/4 in NT only if you are technically inclined and motivated.    Otherwise, you should use the Wave/4 under Win 95/98 and delay upgrading to Win NT until next year when the consumer version is released as Windows 2000.

We highly recommend that you take typical setup precautions before installing the driver and using it:

-	make a back-up copy of your system Registry

-	copy the Wave/4 driver files to a floppy disk.

Driver Installation:

The Plug and Play process in NT is similar to the Windows 95 process.    The Wave/4 hardware is detected automatically and then you are prompted to insert the driver disk.    BUT unlike Windows 95, the hardware resources for plug-in cards (Interrupt IRQ and I/O ports) must be manually selected / assigned.

First step:	if you have already installed a previous version of the Wave/4 NT driver, it MUST be uninstalled.    Follow the steps later in this document in the section, "Removing the Wave/4 NT driver"

Next Steps:	If the Wave/4 card is installed in the PC already, shut down the system and remove the card (this will ensure that the PnP resources are reset).

Follow steps 1-4 below before re-installing the card.

1)	Before booting Window NT, turn on your PC and check your BIOS Setup.    Make sure that the setting for "Using Plug and Play Operating System (OS)" is enabled.    This setting is usually in the PnP / PCI configuration section of the BIOS Setup.

Note:	if you are not sure how to get to your PC's BIOS Setup, then we recommend that you do *not* attempt to use these NT drivers.

2)	Boot Windows NT

3)	Make sure that the Windows NT Plug-and-Play ISA sub-system installed (PNPISA) is installed.

To check this, run the Control Panel, select Devices and make sure 'PnP ISA Enabler Driver' is listed.    The Wave/4 card will not function in an NT system without this support.    The files can be found on the NT 4.0 release CD in the directory:    \drvlib\pnpisa\x86.    To install the PnP ISA support, right-button click on the file, 'pnpisa.inf', and select 'Install'.    (the subsystem files are also found in the same directory location on the SP3 update)

4)	Before installing the Wave/4 card in the system, go to the 'NT Diagnostics' (found in 'Administrative tools' in the Start Menu).    You will need to identify available hardware resources for the Wave/4 card.

- 	The Wave/4 card uses a single IRQ and it can be either IRQ 3, 5, 7, 10, 11, 12 or 15.    Identify an unused IRQ for the Wave/4 card.

- 	The Wave/4 card uses 2 blocks of I/O addresses between 100 hex and 3FF hex.    One I/O block will need to be 8 IO address' long and the other 32 (20 hex) IO address' long. For example: 100-11F (32 IO address') and 120-127 (8 IO address').

-	write down the IRQ and I/O addresses that you identified.

5)	Shut down your PC and install the Wave/4 card in the system.

6)	Boot Windows NT.    As the system starts the 'New Hardware Found…' dialog box will be displayed.    When prompted, place the Wave/4 driver diskette in the floppy drive and install the drivers.

7)	After loading the drivers, a dialog box "No Config Found" will be displayed.    Click OK and another dialog box will appear titled, "Wave/4 Configuration"

8)	Click on the button, 'Set Configuration Manually'.    Set the IRQ and the 2 I/O address ranges to the number and ranges that you identified in step 4 above.

9)	Re-boot the system and the Wave/4 should be ready to go.    In its default configuration, the driver must be started and stopped manually using the Control Panel | Devices each time your PC is booted.    You can change this to automatic as noted below.

Loading the Driver automatically when booting NT:

When first installed, this NT driver requires stopping and starting via the Control Panel each time the PC is booted.    This provides a "safety valve" in case there are difficulties with the driver that could prevent your system from booting.    After you have used the driver for a period of time and you are confident in its operation, you can change it to automatic loading with the following steps:    run the 'Control Panel' and select 'Devices'.    Select 'Gadget Labs Wave/4' and click the 'Startup…' button.    Select 'Automatic' and click the OK button.

Application Setup:

We have tested the driver with many applications including:    Cakewalk Pro Audio 6.01, Cakewalk Pro Audio 7.0, SAW Plus, Sound Forge 4.0, Samplitude 4.5, Cool Edit 96, Goldwave 3.23, and Cool Edit Pro 1.1.    These applications all tested OK with the exception of Cakewalk 6.01 which starts recording but then halts.    (however, the new Cakewalk 7.0 works well.    We do not recommend using Cakewalk 6.01 with NT.)

Regarding settings, the standard setup for Cakewalk 7.0 outlined in the Wave/4 User Guide works fine.    For Cool Edit Pro the settings of 4 seconds and 20 to 22 buffers works. Note that these are guidelines and can very from system to system. You might have to experiment with different values to get things working.

Removing the Wave/4 NT driver:

Follow these steps to remove the Wave/4 driver.

NOTE:    These steps delete files from the main Windows NT folder (directory).    For this example, the directory is named 'windowsNT'.    (make sure you are working in the NT Windows directory and NOT the corresponding Windows 95 directory!)

1.    Disable the Wave/4 driver.    To do this, run the 'Control Panel' and select 'Devices'.    Select 'Gadget Labs Wave/4' and click the 'Startup…' button.    Select 'Disable' and click the OK button.

2.    Remove the Wave/4 INF & PNF files

-    change to the '\windowsNT\inf' folder (directory)

-    find any files that start with the letters 'OEM' with the extensions "INF" or "PNF"    (examples:    OEM1.INF, OEM1.PNF).    View the .INF file with the Notepad program or text editors and look for text that refers to Gadget Labs Wave/4.    If found, delete this file, and it’s corresponding .PNF file.

3.    Delete old driver files

- change to the '\windowsNT\system32' directory:    delete the file:    WAVE4.DLL

(NOTE: You may need to boot into DOS to remove WAVE4.DLL, because it is write protected.)

- change to the '\windowsNT\system32\drivers' directory:    delete the file:    WAVE4.SYS

4.    Remove Wave/4 items from the Registry

To remove items from the NT registry use RegEdt32.EXE which is found in the '\windowsNT\system32' directory. You may also run it by clicking on START, then RUN, and type in REGEDT32.EXE.    After selecting any keys to delete you will need to go to the Security | Permissions menu item and change the protection of the key. For any keys or items you delete you should select 'Full Control' for the Administrator, if the key has sub-keys also select Replace Permission on Existing Subkey. All the pertinent registry keys will reside under HKEY_LOCAL_MACHINE.    The following keys should be deleted:

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Control | Class | {4D36E96C-E325-11CE-BFC1-08002BE10318} | XXXX. Where XXXX is a numeric entry that has the Wave/4 under it.

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Services | Wave4.

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Enum | ISAPNP | GLS0001.

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Enum | Root | LEGACY_WAVE4

Update information

Features now implemented:

1.    MIDI support

2.    The SyncStart feature is implemented for both record and play.    By default it is enabled.    SyncStart assumes that only one application is using the Wave/4 card at a time.

If you would like to "split" the use of the card between 2 applications (as if it was 2 stereo cards), you will need to disable SyncStart.    Here's how:

Using the program, REGEDIT32 change the value of the key,

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Services | Wave4 | Parameter | Device0 | SyncStart

1 = enabled, 0 = disabled.

Reboot NT after making a change or, use the 'Control Panel | Devices' to Start and Stop the Wave/4 driver.

Errata and limitations :

1.	The Wave/4 NT driver does not support 2 Wave/4 cards installed in a PC.    This feature will not be added to the driver.

2.	Sync between channels in Cakewalk 6.01 is not tight (for example, if you play the same audio WAV file through both channels, you can hear some flanging).    Since this is fixed in Cakewalk 7 and 8, we believe that this is an application issue.    Cakewalk no longer recommends using version 6.01 under NT.

Troubleshooting information

Pre-release beta versions of the driver had issues with NT's Plug and Play setup.    Our testing indicates that these issues have been resolved, but for sure, Windows NT plug and play is quite a bit less evolved and stable than Win 95/98.    In case of installation difficulty, here's a bit of troubleshooting info:

Symptoms of Plug and Play not working

-	Checking the registry for the I/O address' and IRQ that are set in the Wave/4 dialog box and they are not there.

-	Software seems to run but there is no input and output, just silence.

-	After exiting and entering the device setup dialog box under Multi-Media | Advanced | Sound Devices | Audio for Gadget Labs Wave/4 the I/O address's are not changed. Also if both are 32 I/O address' long (base + 0x1F), one should be base + 0x1F and the other base + 0x7.

-	if the Wave/4 card is not detected,

first, make sure that the Windows NT Plug and Play support is installed.

second, if the PnP support *is* installed but the Wave/4 card is still not being detected, you may need to change a setting in your PC's BIOS Setup.    Try changing the setting for 'Using Plug and Play Operating System' (this setting is usually found in the PnP/PCI Setup section).    If the setting is currently 'Yes', change it to 'No' or vice versa (if 'No' change it to 'Yes')

third, reinstall the NT PnP ISA support.    To do this, follow these steps:

- delete the registry key: HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\PNPISA

- delete the file 'PNPISA.INF' in the '\windowsNT\Profiles\Administrator\Recent\' directory

- delete the file, 'PNPISA.SYS' in the    '\windowsNT\system32\drivers' directory.    Then reinstall the PnP ISA support as detailed in the Driver Installation section above. (see the section above, 'Driver Installation', step 3)

-	If the card is detected but the card does not function, you may need to setup the IRQ and I/O addresses manually in the Registry with the program REGEDT32.    First run the Control Panel and note the resources used by the Wave/4.

Then, run Regedt32.EXE and if the addresses are different, then change them.

NOTE:    on some systems, for some weird reason the I/O addresses get swapped.    Try exchanging them.

The resource info for a Wave/4 card can be found at:

HKEY_LOCAL_MACHINE | System | CurrentControlSet | Services | Wave4 | Parameter | Device0

There are 3 parameters under this key for resources:

Port – the SoundCache audio port, addressed from 0x100 to 0x3E0 in 0x20 steps.

MIDI Port – the MIDI port, addressed from 0x100 to 0x2F0 in 0x10 steps.

Interrupt (IRQ) - 3, 5, 7, 10, 11, 12 & 15.

-	another PnP trick:    first, install the Wave/4 card with Windows 95.    Then, run the Control Panel Device Manager (under System).    In the device list, under 'Sound, Video and Game controllers', find the device line for the Wave/4 card.    Click on the 'Properties' button and then view the card's 'Resources'.    Note the resources that were assigned by Windows 95.    Then, boot NT and try putting these same resource values into the Registry setting listed above.